Wave Transmission Over and Through Rubble-Mound Breakwaters

John P. Ahrens

Contract Report submitted to US Army Engineer Research and Development Center, Coastal and Hydraulics Laboratory, Vicksburg MS, 2001.

Introduction

Wave transmission is usually the most important performance characteristic of breakwaters. There are two basic kinds of breakwaters: rubble-mound and vertical or caisson, with a variety of hybrid types. This paper will be primarily concerned with wave transmission over and through rubble-mound breakwaters.  Breakwaters can also be categorized on the basis of their height and their permeability.  Cross sections of the “classical” multi-layered rubble-mounds are shown in the SPM (1984) Figures 7-116 and 7-117 and as Fig. 1 in this test; these structures generally have complex cross-sections with relatively impermeable cores (because it contains fine material).  Reef breakwaters represent the other extreme with simple cross-sections and a very permeable core.  From the stand point of modeling wave transmission characteristics, reef breakwaters represent a simpler problem than multi-layered structures.

Reef breakwaters are a homogeneous pile of stone with individual stone weights in the range of those ordinarily used in the armor or first underlayer of  multilayered breakwaters. Low-crested versus high crested rubble-mounds is probably a less desirable way to categorize breakwaters.  The same structure that may perform as a low-crested breakwater at high tide during a storm, can perform as a high-crested breakwater at low tide during a calm period.  This paper will provide the background information on wave transmission characteristics of rubble-mound breakwaters and lead up to models to predict performance.

Background

This paper will use the data on reef breakwaters given in Ahrens (1987a) as the basis of developing models for predicting wave transmission characteristics for rubble-mound breakwaters. The reason for this choice is because it is the data set the author is most familiar with, the data set is large (204 tests) and a wide range of irregular wave conditions were employed, and the test setup appears superior to other studies.  Predictions developed from the Ahrens data set will be compared to data from other studies.  A brief description follows of studies that contributed to the analysis in this paper.

Breakwater Transmission Study of Seelig (1980)

This extensive laboratory study looked at wave transmission for a large number of breakwaters.  Some of the breakwaters tested were designed to provide insight on various aspects of wave transmission and would not be viable structures, e.g., one breakwater was an aluminum trapezoid.  Unfortunately, most tests were with monochromatic waves.

The data of Seelig’s (1980) used in this paper are: 1.) only his tests with irregular waves, 2.) no tests were considered with the structure at the top of a 1 on 15 fronting slope, since this setup makes it difficult to estimate the incident wave conditions, and 3.) only breakwaters constructed predominately with stone were considered.  This filtering criteria left data from 6 breakwaters or 117 tests available for evaluation.  Seelig’s tests were at a relative large scale for a laboratory study, although no scale was assigned to the tests.  Water depths at the toe of the structures ranged from 45 – 85 cm and many tests had incident significant wave heights in the range 15 – 20cm. The following table summarizes some of the important variables of the tests conducted by Seelig and used in this paper.

Table 1, summary of Seelig (1980) data.

| Seelig’s Desig-nation | No. Of Tests | Crest Height Structure hc(cm) | Cross- Sectional Area, At(cm2) | Median Dia-meter Armor d50(cm) | Median Dia-meter Core, d50(cm) | Equi-valent Dia-meter d50(cm) | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 20 | 66 | 11,570 | 11.1 | 2.9 | 3.75 | trapezoid |
| 4W | 7 | 66 | 11,570 | 11.1 | 2.9 | 3.75 | trapezoid, with plate |
| 5 | 30 | 33 | 2,954 | 11.1 | 2.9 | 4.91 | trapezoid |
| 6 | 15 | 60 | 2,700 | 2.9 | N.A. | 2.9 | crib, no core |
| 9 | 21 | 70 | 9,450 | 11.1 | 2.9 | 3.31 | trapezoid, cap & Dolos |
| 10 | 24 | 66 | 8,514 | 16.1 | 2.9 | 3.60 | trapezoid |

All of the above structure were two layers except Seelig – 6 which was composed of three rectangular cribs, using fine wire to hold in the stone.  Seelig – 9 was two layers, but the armor was partly stone, partly Dolos and a small impermeable cap.  Equivalent diameter was used to relate Seelig’s structures to the reef structures tested by Ahrens (1987) and the method used to compute this diameter will be given in the analysis section.

van der Meer’s Reef Breakwater Tests

The transmission tests of van der Meer were conducted as part of a larger study at Delft which included quite a lot of effort related to rubble-mound stability, van der Meer (1988).  All tests used irregular waves, structures in 40cm of water, a trapezoidal reef configuration (no core), crest width 30cm, and a median stone diameter of 3.44 cm.  Wave heights and periods were roughly in the range of 8 – 20cm and 1.9 - 2.6 sec. respectively.

Table 2, summary of van der Meer (1988) data.

| No. of Tests | Crest Height, hc(cm) | Cross-Sectional Area, At(cm2) |
| --- | --- | --- |
| 11 | 40 | 4,400 |
| 9 | 53 | 7,208 |
| 11 | 30 | 2,700 |

Data was summarized from tables and figures in Daemen (1991).

Daemen’s (1991) Low-Crested Breakwater Wave Transmission Tests

Daemen’s (1991) study was also conducted at Delft.  The same trapezoidal, two layer rubble-mound was used for all tests.  The crest height was 40cm, crest width was 34cm, and the cross-sectional area was 3,560cm2.  Incident wave heights ranged roughly from 3 – 15cm and wave periods from roughly 1.0 – 2.9sec.  Table 3 summarizes important information about Daemen’s tests.

Table 3, summary of Daemen (1991) data.

| No. of Tests | Range of Toe Water Depths, ds(cm) | Armor Median Diameter, d50(cm) | Core Median Diameter, d50(cm) | Equivalent Diameter, d50(cm) |
| --- | --- | --- | --- | --- |
| 47 | 20.4 – 44.0 | 4.0 | 2.8 | 3.26 |
| 6 | 41.0 – 45.7 | 6.1 | 2.8 | 3.64 |

Wave Transmission Tests at HRS Wallingford

There are two studies frequently cited from HRS Wallingford: Allsop (1983) and Powell and Allsop (1985).  Data is plotted in these two references, but not tabulated.  Daemen (1991) tabulates some data from these references but cites unresolved and unspecified concerns about the wave conditions.  Eight structures with trapezoidal cross sections were tested.  Six structures were reef types in that only one stone size is used in a homogenous cross section, however, the prototype stone is quite large, 10 – 15 tons, and test conditions suggest that the prototype site has a high tide range.   A seventh structure is a structure capped with 17 –25 ton stone and the remainder of the structure 10 –15 tons. The eighth structure is a relatively complicated, three layer structure.   Data from HRS Wallingford is plotted in Fig. 2 and tests using the three layer structure is denoted ‘Multi-layer’ in the legend.

Two aspects of Fig. 2 are interesting: 1.) There is surprisingly little scatter in Kt plotted versus the relative freeboard, F/Hs, where F, the freeboard of the structure, is the difference (hc – ds) between the crest height of the structure, hc , and the water depth, ds, at the toe of the structure and Hs is the incident significant wave height. and 2.) Kt is surprisingly low for high structures, F/Hs > 2.0, indicating that there is little wave energy propagating through the multi-layer breakwater.

Details related to the test setup at HRS Wallingford and information related to laboratory stone sizes and cross sectional areas of structures would be difficult to estimate.  Daemen (1991) notes that in obtaining this data from HRS that it was designated ‘confidential’.  Daemen also questions the reliability of the wave data.  The author is concerned about the low scatter about the data trend, most studies do not show such clean trends.

Reef Breakwater Study of Ahrens (1987a)

This study investigated the stability, wave transmission, wave reflection and energy dissipation characteristics of reefs breakwaters, Ahrens (1987a), Ahrens (1987b),  Ahrens and Fulford (1988),  Sheppard and Hearn (1989), and Ahrens and Cox (1990). The primary goal of this study was to determine the cost effectiveness of this type of breakwater as shoreline protection. Max Sheppard and Janet Hearn were IPA’s  at the U. of Florida that worked with the reef breakwater data to provide an independent appraisal of the reef breakwater study.  Janet Hearn got her MS on this subject and Max Sheppard was her thesis advisor. Table 4, summarizes important information about these tests.

Table 4, summary of Ahrens (1987a) data.

| Subset No’s | No. of Tests | Water Depth At Toe, ds(cm) | Range of Crest Heights At Equilibrium, hc(cm) | Medium Stone Diameter, d50(cm) | Cross Sectional Area of Structure, At(cm) |
| --- | --- | --- | --- | --- | --- |
| 1 & 2 | 30 | 25 | 15.9 – 24.6 | 1.86 | 1,170 |
| 3 & 4 | 41 | 25 | 17.4 – 29.5 | 1.86 | 1,560 |
| 5 & 6 | 52 | 25 | 19.5 – 35.9 | 1.86 | 2,190 |
| 7 & 8 | 64 | 25 | 24.3 – 32.8 | 2.93 | 1,900 |
| 9 & 10 | 18 | 30 | 25.0 – 32.1 | 2.93 | 1,900 |

Tests were run to determine either stability or wave transmission characteristics.  Stability tests ran waves until the structure reached equilibrium in terms of deformation to wave action.  Other wave conditions were run with the deformed structure to determine wave transmission characteristics.  Wave heights and peak periods ranged roughly from 1 – 18cm and 1.45 – 3.60sec. respectively. The structure was surveyed after each wave condition to document shape, crest height, etc.  All tests run primarily to determine stability started with a standard trapezoidal shape.  After severe wave conditions the original trapezoidal shape was quite ‘rounded’, see Ahrens (1987a) and Ahrens (1987b).  Fig. 3 shows a cross-sectional profile of a reef breakwater as built and after being damaged by wave attack.  Both stability and transmission tests yielded valid transmission data.  In developing a model to predict wave transmission Ahrens (1987a) assumed that the damaged structure shape could be characterized knowing only the equilibrium crest height, hc, and the cross sectional area, At and not the detailed shape.  Analysis indicates that this assumption was reasonable, Ahrens (1987a) and Cornett, et al, (1993), therefore this approach will be followed in this paper.

Simple Concepts

The dominant modes of wave transmission can be identified as a function of relative freeboard, F/Hs .  If the structure is submerged, F/Hs < 0, the dominant mode is wave transmission over the crest.  If  0 < F/Hs < 1.0 the dominant mode of transmission is by wave runup and overtopping. If the structure is high, say F/Hs > 2, transmission is primarily through the breakwater, see Fig. 4.  If wave conditions are moderate a surprisingly large percent of the incident wave energy can transmit through a reef breakwater.

The parameter F/Hs, is particularly useful for showing the wave transmission characteristics of rubble-mounds.  This is because wave transmission is primarily over the crest if the structure is submerged or by overtopping for 0 < F/Hs < 1.0.  For submerged rubble-mounds the larger the wave the less efficient the transmission process, i.e.,  Kt  decreases as F/Hs increases.  For rubble-mounds with a crest somewhat above the still water level the efficiency of the transmission process increases as the wave height increases, i.e., Kt increases as F/Hs decreases.  The two trends noted above compliment each other so that (in some data sets) they form a reasonably  compact scatter band of Kt data over a wide range of F/Hs, see Fig 2 showing laboratory data from HRS Wallingford.  Unfortunately, the trends shown in Fig. 2 are misleadingly simple and apparently ignored the influence of other variables which are important, Daemen (1991).  This problem will be addressed further, below.

Approach

The approach used for this paper is to develop a parameterization and prediction equation for each of the fundamental modes of transmission for reef breakwaters: transmission by overtopping of surface piercing structures, transmission over the crest of submerged structures, and transmission through for both surface piercing and submerged structures.  Comparisons will be made to the data sets discussed above.  The method of analysis will be similar to Ahrens (1987a) except the observed wave transmission will be defined in the standard way, i.e.,

Kt = Ht/Hi ,									(1)

where Kt is the transmission coefficient, and Ht and Hi are the incident and transmitted wave height respectively, see Fig.2. In Ahrens (1987a) the transmission coefficient was defined:

Kt(Ahrens) = Ht/Hc ,								(2)

where Hc is a wave height determined during a calibration phase of the Ahrens (1987a) study. Hc  is the wave height that was observed in the wave channel at the location of the transmitted wave gages when the structure was not in the channel, for a given value of Ht.  It was felt that Kt(Ahrens) might be a better indication of a reef’s performance that the traditional transmission coefficient, Eq. 1.  Eq. 2 will yield a transmission coefficient roughly 3 – 5 % higher than Eq. 1.  All 204 tests conducted by Ahrens were with irregular waves so the zero-moment wave height is used in the calculations.

In Ahrens (1987a) there is a complete set of equations to estimate Kt(Ahrens) over a wide range of conditions.  Wave tank tests of ‘tandem reef’ breakwaters conducted at NRC Canada found that the wave transmission predictions given by Ahrens fit their data quite well, Cornett, et al. (1993).  Tandem breakwaters are submerged breakwaters placed in front of a surface piercing breakwater to reduce the severity of wave attack on the larger structure.

One of the basic premises of the analysis is that a relatively simple set of variables , that could be expected to be known, can be used to make good estimates of the transmission coefficient.  This set of variables is:

Hmo or Hs  -  incident wave height

Tp  -  wave period of peak energy density

ds  -  water depth at toe of structure

At  -  total cross-sectional area of breakwater

hc  -  crest height of structure

d50  -  median size of stone in reef

For reef breakwaters the above set of variables works very well, but for more complex breakwaters some obvious problems arise.

This approach assumes that the predicted transmission coefficient, of a hybrid type, can be defined and that it will give reasonable estimates of coefficients measured in the laboratory and hopefully the field.  The following transmission coefficient is defined:

Kt  [ (Kt(over))2 + (Kt(thru))2] ,						(3)

where Kt(over) is the transmission coefficient associated with runup on surface piercing breakwaters or transmission over the crest of a submerged breakwater and Kt(thru) is the transmission coefficient associated with wave energy transmitting through the structure.  These efforts to put these pieces together will provide important insight to the physical processes that should permit reasonable estimates of transmission coefficients over a wide range of conditions for reefs and by comparison to other data sets to help understand the performance other rubble-mound structures.

In addition to a ‘dominant modes’ of transmission approach, a surprisingly effective approach based on the relative height and width of the structure will also be developed.

Analysis

The primary analysis technique used in this paper is a combination of dimensional analysis and regression analysis based on experience and (hopefully good) physical insight.  This analysis is used to develop equations to predict transmission coefficients.  These equations represent what the author regards as the “best” of many investigated.  Best means an equation which has a high correlation,  is relatively simple, is consistent with the physics of the process as it is currently understood, and approaches logical limiting values.

Transmission through a Reef

To determine the amount of wave energy that transmits through the void spaces in the stone of a rubble-mound it is necessary to focus on tests of structures with high values of F/Hs.  You want a value high enough so that virtually all the transmitted wave energy measured came through the structure and at the same time there are enough data to allow determination of the importance of different variables.  These joint considerations create a ‘catch 22’ situation in selecting a cut-off value for F/Hs, see Table 5.

Table 5, relationship between relative freeboard and number of           tests available for analysis

| Relative Freeboard | Number of Tests |  |
| --- | --- | --- |
| F/Hs > 1.0 | 37 |  |
| F/Hs > 2.0 | 14 |  |
| F/Hs > 3.0 | 7 |  |

In Ahrens (1988) the cut-off value was F/Hs > 1.0.  Reanalysis indicates that this level is probably too low, i.e., there is overtopping wave energy ‘contaminating’ the portion transmitted through the structure.

For this analysis the data used satisfied F/Hs > 2.0 which eliminated all but 14 tests.  The relation developed from this data is:

Kt(thru) = 1.0/(1.0 + f1 ),  where f1 = (Hs/d50)0.982exp[ 0.433+2.35(At/Lohc)],   (4)

Eq. 4 fits the data quite well with an R2 = 0.978.  A fit this good suggests that Eq. 4 may have the physics about right, which is quite promising since the transmission through must be subtracted from the total transmitted energy to determine the portion  transmitted over the crest by overtopping or determine the portion transmitting over the submerged crest.

In Eq. 4 the term, Hs/d50 , indicates that when the scale of the disturbance is large compared the size of the void spaces in the stone which transmit the wave energy the transmission process is inefficient.  This trend may seem paradoxical when compared to the process of transmission by overtopping, where transmission is more efficient as the wave height increases.  It is the reversal of the influence of Hs for surface piercing rubble-mounds which makes analysis difficult and makes the definition for the transmission coefficient, such as Eq. 3 useful. Research on steady flow through porous media indicates that flow through stone larger than about 1.0 cm will be in the turbulent range and that d15 is better than d50 in characterizing the actual size of the void space, van Gent (1993), but d50 is typically used.  van Gent’s observations indicates that laboratory tests should measure the size distribution of the stone used in the rubble-mound.  In Eq. 4 the term, At/hc , is a measure of the structure’s width and is normalized by the deep-water wave length.  Thus the term, At/Lohc , brings in the influence of the structure’s width and the wave period.

Appendix 1, Regression Analysis, shows the statistics associated with the regression analysis for Eq. 4.  The “t” statistic (Student’s t) for the, Hs/d50 , variable is quite high, 21.5, indicating that there is very little chance that its contribution to predicting Kt(thru) occurred by chance.  The t statistic  for the variable, At/Lohc , is 6.2, which is also high enough to indicate that the probability is low that the influence of the variable occurred by chance.  The P-value statistic shown is an estimate of the probability that the influence of a variable occurred by chance.  Unfortunately, these statistical parameters do not understand physics and the sample size is small, i.e., n = 14; both these limitations should mitigate one’s enthusiasm for the high correlation shown.

In the earlier analysis by Ahrens (1987a) the relation developed for transmission through the reef was:

Kt(thru) = Ht/Hc = 1.0/(1.0 + f2 ), where f2 = (HsAt/Lpd502)0.592 ,		(5)

Lp is the local wave length associated with the peak period.  Eq. 5 predicts a slightly different transmission coefficient than Eq. 4.  The analysis of Ahrens indicated that f2 was a particularly effective parameter to characterize transmission through a reef breakwater and this conclusion was apparently confirmed by the analysis of  Sheppard and Hearn (1989).

To provide an independent test of Eqs. 4 and 5 they are compared to data of Seelig (1980), van der Meer (1988), and Daemen (1991), which are discussed above.  The comparison is shown in Table 6 and Fig. 5.; Fig. 5 compares data only to Eq. 4.  Only three tests of Seelig, five tests by Daemen  and none of van der Meer’s had relative freeboards  greater than 2.0, F/Hs > 2.0,  so a slightly lower standard , F/Hs > 1.5, was used to compare predicted and observed values of Kt(thru).  This slight lower standard allowed comparison of 23 tests with a variety of configurations.

Table 6, comparison of predicted and observed Kt’s for relatively high breakwaters, i.e., F/Hs > 1,5

| Relative Freeboard F/Hs |  | Observed Kt | Predicted Kt(thru) Eq. 4 | Predicted Kt(thru) Eq. 5 | Principal Investigator |
| --- | --- | --- | --- | --- | --- |
| 4.000 |  | 0.118 | 0.163 | 0.208 | Daemen |
| 3.134 |  | 0.213 | 0.224 | 0.241 | Seelig - 10 |
| 3.015 |  | 0.117 | 0.150 | 0.193 | Daemen |
| 2.909 |  | 0.064 | 0.140 | 0.224 | Daemen |
| 2.857 |  | 0.381 | 0.398 | 0.332 | Seelig - 10 |
| 2.778 |  | 0.217 | 0.164 | 0.185 | Seelig - 9 |
| 2.279 |  | 0.105 | 0.128 | 0.176 | Daemen |
| 2.108 |  | 0.107 | 0.127 | 0.176 | Daemen |
| 1.927 |  | 0.155 | 0.154 | 0.198 | Seelig - 10 |
| 1.903 |  | 0.107 | 0.123 | 0.174 | Daemen |
| 1.882 |  | 0.133 | 0.162 | 0.220 | Daemen |
| 1.748 |  | 0.167 | 0.114 | 0.152 | Seelig - 9 |
| 1.736 |  | 0.113 | 0.102 | 0.132 | Seelig - 4 |
| 1.707 |  | 0.126 | 0.142 | 0.192 | Seelig - 10 |
| 1.694 |  | 0.071 | 0.040 | 0.095 | Seelig - 4 |
| 1.694 |  | 0.109 | 0.074 | 0.114 | Seelig - 4 |
| 1.684 |  | 0.087 | 0.176 | 0.206 | v.d. Meer |
| 1.680 |  | 0.098 | 0.114 | 0.150 | Seelig - 10 |
| 1.655 |  | 0.049 | 0.090 | 0.174 | Daemen |
| 1.543 |  | 0.098 | 0.077 | 0.109 | Seelig - 9 |
| 1.543 |  | 0.059 | 0.139 | 0.174 | v.d. Meer |
| 1.534 |  | 0.147 | 0.103 | 0.148 | Seelig - 9 |
| 1.515 |  | 0.117 | 0.147 | 0.231 | Daemen |

.Table 6 shows predictions of Kt  by Eq. 5 are somewhat high.  Eq. 5 is expected to be high because it is predicting Ht/Hc not Ht/Hi .  Eq. 4 is a better predicter than Eq. 5.  Observed values in Table 6 may be slightly low for tests by Daemen (1991) because of shoaling between the wave gages and the structure.  Linear shoaling suggests that the incident wave height may be under estimated by roughly 3 %.  This same condition may explain why van der  Meer’s observed values seem low, but the details of his test setup are not known.  In addition, Daemen’s tests were with a two layer rubble-mound, but since the core material had d50 = 2.8 cm and armor with d50 = 4.0 cm this should not be an important difference, i.e. this breakwater should be similar to a reef since the core stone is relatively large. For breakwaters with two layers the equivalent stone diameter was used in Eqs. 4 and 5.   The equivalent diameter is calculated as follows:

d50(equivalent) = (At/Bn(eqivalent)), 					(6)

where Bn(equivalent) is the equivalent bulk number, given by,

Bn(equivalent) = Ac/d502(core) + AA/d502(armor),

where Ac and AA are the cross sectional area of the armor layer and core respectively and d50(core) and d50(armor) are the median diameters of the core and armor stone respectively.  The bulk number was found to be a useful parameter by Ahrens (1987a) and also by Dutch researchers.  The equivalent diameter should be useful for stone sizes greater than about 1.0cm where the flow is in the turbulent range, van Gent (1993).

The linear correlation between the observed  Kt  given in Table 6 and Eq.4 is 0.83 and between Eq.5 is 0.64, indicating that Eq. 4 is the better predictor.  Considering the difficulties in comparing the results of studies conducted by different principal investigators and in some case different laboratories, Eq. 4 appears to give good results.

Transmission by Overtopping of Surface Piercing Rubble-Mounds

This part of the analysis will look at the 98 tests of Ahrens (1987a) that fall in the range, 0 < F/Hs < 2.0.  These are the tests where wave transmission is dominated by wave runup and overtopping generating waves on the lee-ward side of the breakwater.  Before the regression analysis was performed the estimated wave energy transmitted through the breakwater was subtracted out using Eq. 4.  The “best”  regression equation to predict this component of wave transmission is:

Kt(over) = 1.0/(1.0 + f3 ),  							(7)

where f3 = exp[ 0.465+12.7F/(HsLo) – 17.4/(At/hcd50) + 0.00118/(Hs/Lo)] .

Eq. 7 does a pretty good job predicting this portion of the wave transmission with R2 = 0.930. i.e., explains 93 % of the variance.  The dominant variable in Eq. 7 is a relative freeboard type variable, F/(HsLo), where (HsLo) is a term proportional to the potential runup on the structure such as given by Hunt’s equation, i.e., R = (HsLo) tan, where R is the runup and tan is tangent of the structure slope.  Allsop (1983) also showed that F/(HsLo) was a good predictor of wave transmission coefficients for surface piercing rubble-mound breakwaters with cores that blocked transmission through the structure, see Fig. 2; this is the same type of situation as being discussed here.  The, At/hcd50 , term in Eq. 7 is the relative width of the breakwater in stone diameters. Clearly the width of the structure has some influence on wave transmission, although not as much as the freeboard, to the extent that the two can be distinguished.  The wave steepness term, Hs/Lo , in Eq. 7 is a little hard to explain; the implication is that as steepness increases wave transmission increases.  This trend is somewhat counter to the relative freeboard term, but may be necessary to balance some effect in that term or possibly to balance some effect introduced by using Eq. 4 to subtract out the transmission through portion of wave energy.  Still, the wave steepness has high t statistics (see appendix) and must be included to get an R2 above 0.90.

The following four figures compare predicted and observed or estimated Kt’s for surface piercing rubble-mounds, using data not used in the development of the equations:  Fig. 6 shows the predicted Kt(over) using Eq. 7 versus the estimated Kt(over) obtained by subtracting the estimated energy transmission through the structure using Eq, 4 from the observed Kt  from the Delft studies for,  0   F/Hs < 2.0.  Fig. 7 shows the predicted Kt using both Eqs. 4 and 7 compared to the observed Kt from the Delft studies for, 0   F/Hs.  Fig. 8 shows the predicted Kt(over) using Eq. 7 versus the estimated Kt(over) obtained by subtracting the estimated energy transmission through the structure using Eq, 4 from the observed Kt  from Seelig’s (1980) studies for,  0   F/Hs < 2.0.  Fig. 9 shows the predicted Kt using both Eqs. 4 and 7 compared to the observed Kt from Seelig’s (1980) studies for, F/Hs  0 .

In general, Figures 6 through 9 show that the prediction equations do a better job of predicting Seelig’s (1980) than the data from the two Delft studies, van der Meer (1988) and Daemen (1991).  For some reason the prediction equations consistently over predict Kt’s for the Delft studies.  The prediction equations do a relatively good job of predicting Seelig’s  observed Kt data for surface piercing rubble-mounds.  The reasons for the above inconsistencies is not known, but may be almost ‘buried’ in the details of the test setups or procedures.  There may be some value in studying the analysis of wave transmission given in van der Meer and d’Angremond (1991) or van der Meer and Pilarczyk (1995).  The Dutch seem to have attacked the wave transmission problem on two levels, possibly three: 1.) A design level which is a statistical approach based on confidence bands around a lot of Kt data plotted against the relative freeboard, F/Hs  and 2.) A more detailed analysis which uses dimensional analysis and tries to understand the physics of the process, e.g. van der Meer and d’Angremond or van der Meer and Pilarczyk.  In my opinion Level 1 is too crude and seems to simply ignore wave transmission through rubble-mounds.  Level 2 is more complex and may consider wave transmission through a rubble-mound.  Level 3 would be focused research by Dutch and other EU researches to study the wave transmission problem through porous media  using a combination of laboratory tests and theoretical considerations to specifically address this problem at a fundamental level, e.g., van Gent’s (1993).  However, van Gent’s research may have been directed at solving numerical modeling problems of berm breakwater stability.

Wave Transmission Over a Submerged Rubble-Mound

The approach  is to use  Eq. 4 to subtract out the transmission through a submerged breakwater and then use regression, as above, to determine the parameterization and prediction of the transmission over the crest. However, it is not clear how to subtract out the transmission through, out of the total transmission.  The approach adopted was to use the portion of wave energy transmitting under a plate projecting down into the water from above as a weight factor for the transmission through.  The equations for idealized  transmission over and under an impermeable plate are given in Wiegel (1964), but are commonly referred to as Fuch’s Equation.  This procedure yielded the relatively small effective Kt(thru), order Kt(thru)  0.1 , that would be expected for a submerged rubble-mound.

Regression analysis for the remaining energy yielded the following equation:

Kt(over) = 1.0/(1.0 + f4 ),  where f4 = exp[ -0.646 +0.631F/Hs + 0.00137Bn] .  (8)

The relative freeboard term in Eq.8 was expected to be important for submerged rubble-mounds and as expected had a high t-statistic, i.e., t = 13.8.  Remember the freeboard is negative for a submerged structure !  The bulk number, Bn , in Eq. 8 is a measure of the size of the reef,  Bn = At/d502, and also had a high t-statistic, i.e., t = 8.4.  Even though Eq. 8 makes physical sense it’s R2 = 0.73 is none too good and indicates that our understanding is none too good.  Both the Dutch and HRS research confirm the findings given in Eqs. 7 and 8, i.e., that F/Hs  is an important variable for submerged rubble-mounds and the other freeboard variable, F/(HsLo), is better for predicting Kt for surface piercing rubble-mounds, Allsop (1983) and van der Meer and d’Angremond (1991).

Figs. 10 and 11 show the predicted Kt using both Eqs. 4 and 8 versus the observed Kt for Seelig’s (1980) and Dutch studies of van der Meer (1988) and Daemen (1991) respectively.  In Fig. 10 the predicted values follow the trend of Seelig’s data pretty well, albeit with considerable scatter.  Also denoted in Fig. 10 are the tests (test series BW -  4W) of a rubble-mound breakwater with an impermeable plate in it; as expected the prediction method over predicts the Kt’s for this test series.  In Fig. 11 the predicted Kt values show a much greater range than the observed values, with a tendency to over predict Kt’s for relatively high structures and under predict for lower structures.  The problem indicated in Fig. 11 needs to be investigated further.

Summary of ‘Dominant Mode’ Analysis

Eq. 4 was used to estimate Kt(thru) for the 14 reef breakwater tests of Ahrens (1987a) where F/Hs > 2.0.  Eqs. 4 and 7 were used together in Eq. 3 to estimate Kt for the 98 reef breakwater tests where, 0 < F/Hs < 2.0.  Eqs. 4 and 8 were used together in Eq. 3 to estimate Kt for the 92 reef breakwater tests where F/Hs < 0.  In addition, for submerged reefs a weight function was applied to Eq. 4 based on linear wave theory for wave transmission under a rigid, impermeable plate, Wiegel (1964).  The ratio of predicted Kt’s for this method to the observed Kt is shown in Fig. 12 as a function of F/Hs.  Fig. 12 shows that almost all estimates are better than  20%.  The root mean percent error for the 204 tests is 9.6% and the linear correlation between predicted and observed Kt’s is 0.969.

Transmission Analysis Based on only Structure Characteristics (H – W approach)

A surprising aspect of the analysis was the discovery that reasonably good predictions of Kt could be made for all 3 data sets, over the entire range of conditions using just two variables that included only the structure characteristics, i.e., did not include Hs or Tp .  As usual the basic prediction equation is of the form:

Kt = 1.0/(1.0 + fs) , where fs = f(hc/ds, At/hcd50).				(9)

In Eq. 9 the terms, hc/ds, and ,At/hcd50, characterize the relative height and relative width of the structure respectively.  The basic functional form is referred to as Hoerl’s equation which is frequently used in regression analysis and is given, in general, by:

f(Hoerl’s) = exp[C0 + C1X1]X2C2 , where the X’s can be any independent variable, Daniel and Wood (1980). Hoerl’s function is useful because it’s generality allows a fit to a lot of quite different data trends and it is linerizable (of course the form of Eq. 9 is designed to approach the logical limits of  0 and 1.0. For two variables, e.g., there are 9 simple permutations, corresponding to the variable, the reciprocal of the variable, and the logarithm of the variable.  Relative freeboard, F/Hs, would be less useful for this analysis since it is negative for submerged rubble-mounds and power law relationships are precluded. The following equations are the ‘best’ fit equations for each of the 3 major data sets:

fs = exp[-2.141 + 3.221(hc/ds) – 17.83/(At/hcd50)] , Seelig (1980)		(10)

fs = exp[3.933 – 4.665/(hc/ds) + 0.0491(At/hcd50)] , v.d. Meer (1988)	(11)

and Daemen (1991)

fs = exp[-2.522 + 3.399(hc/ds) – 25.21/(At/hcd50)] , Ahrens (1987a)		(12)

Interestingly in none of the above equations, (Eqs. 10, 11, or 12), did a power law relationship for either variable contribute to the best fit.  Table 7 summarizes relevant information about this analysis.

Table 7, summary of regression analysis based only on structure characteristics.

| P. I. | Eq. no. | number of data | R2 | variable, hc/ds |  | variable, At/hcd50 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | range | t – stat. | range | t – stat. |
| Seelig | 10 | 110 | 0.897 | 1.56 – 0.44 | 21.8 | 15.5 – 46.7 | -6.1 |
| v.d. Meer and Daemen | 11 | 84 | 0.798 | 1.96 – 0.76 | -14.7 | 24.4 – 40.1 | 3.30 |
| Ahrens | 12 | 204 | 0.797 | 1.44 – 0.64 | 27.3 | 19.8 – 60.2 | -10.3 |

Figure 13 gives a visual impression of the range of relative heights and widths considered for the three studies.

Examples

To illustrate the implications of the analysis based only on structure characteristic two examples are given for conditions typical of where the equations might be used.

Example 1:  Various structure sizes are built in a water depth of 4.0m.  The structures are trapezoidal with slopes of 1 on 2 and heights ranging from 2.0m to 6.0m. The stone size is d50 = 0.4m or about 170kg.  The width variable, At/hcd50 , ranges from 13.0 (stone diameters thick) at hc/ds = 0.5 to At/hcd50 = 33.0 at hc/ds  = 1.5.  Figure 14 shows the predicted values of Kt as a function of the relative height, hc/ds .  What is curious about Fig. 14 is that the Dutch data, Eq. 11,  predicts higher Kt’s for low structures and lower Kt’s for high structure than would be expected from the other two studies.  The relatively high values of Kt for high structures suggests why Ahrens (1987a) is so concerned with transmission through the structure.  The surprisingly low values of Kt from the Dutch study for low structures, such as tandem breakwaters suggests, contrary to observation Cornett, et al. (1993),  that tandem breakwaters would not be very effective.

Example 2:  This is a single structure 4.0 high in water with a mean depth of 4.0m, but with a tide range of 3.0m.  This tide range causes a range of relative depths of, 0.73  hc/ds  1.60. The structure is other wise similar to Example 1, i.e., cot = 2.0 and d50 = 0.4m.  The width variable is At/hcd50 = 23.0.  Fig. 15 shows the predicted Kt as a function of relative structure height; as expected the Dutch study shows the lowest Kt for the lowest relative structure height.  Also, as in Example 1, the data of Ahrens (1987) shows the highest Kt’s for high structures.

Characteristics of Transmitted Waves

One of the surprising findings of the reef breakwater study was that the incident period of peak energy density, Tp, was conserved in the transmission process, regardless of whether the rubble-mound was submerged or surface piercing, Ahrens and Cox (1990).  However, this surprising characteristic was not true of the average up-cross period, Tz .  The modification of the average period is strongly correlated with the relative freeboard, F/Hs, and the dominant modes of transmission.  A submerged rubble-mound will induce wave breaking and this process will generate harmonics of the incident frequency causing a reduction in the average wave period.  For quite low structures, say F/Hs  -3, the reduction in wave period is roughly 10 %, but for structures say F/Hs  -0.5 the reduction can be in the range of from 20 to 40%.  For relatively low, but surface piercing rubble-mounds where transmission is dominated by overtopping, say F/Hs  0.5, the reduction in the average period is in the range of 25 to 50 %.  Of course overtopping waves impact the water on the lee side generating harmonics, rather similar to the waves breaking over the submerged rubble-mound, which reduce the average wave period.  When F/Hs  1.0 the average period transmitted is about the same as the incident wave period.  For F/Hs  1.5 and greater, the structures acts like a filter to filter out the higher frequency components in the incident spectrum, causing the average transmitted period to increase somewhat;  typically this increase is in the range of 10 to 20%.

There is considerable discussion of transmitted wave characteristics in Sheppard and Hearn (1989) for the data collected by Ahrens (1987).  Seelig (1980) discusses changes in the joint distribution of height and period between incident and transmitted waves.  Seelig notes that the major difference between the joint distributions of incident and transmitted waves is that the mean period of the smaller waves is lower for the transmitted waves than for the incident waves.

William Allsop is concerned about the characteristics of transmitted waves and would like to link up with someone with money to study this problem in the laboratory (personal communication 2000).  Jentsje van der Meer presented a paper, maybe papers, on this topic at ICCE ‘2000 so we should be looking for this in proceedings (personal communication 2001).  No analysis was performed on this aspect of wave transmission because the data is no longer available.

Summary and Conclusions

This paper examines a number of studies of wave transmission over and through rubble-mound breakwaters. The purpose was to try and gain improved understanding of the phenomenon so as to define the present ability to predict Kt’s, hopefully to develop improved prediction techniques, and determine research needs.

An approach to understanding the wave transmission process better based on the ‘dominant mode’ of transmission is outlined.  This outline indicated that the approach is promising, but is not foolproof.  Dominant mode approach would benefit from some carefully conducted laboratory tests.  The author believes the dominant mode approach to be preferable to the approach to predicting transmission coefficients given by van der Meer and d’Angremond (1991).  The analysis in this paper also shows that details related to the geometry of the structure may not be necessary to make reasonable estimates of transmission coefficients.   Analysis also suggests that two layer rubble-mounds may be treated similar to a one layer (reef breakwater) rubble-mounds by using an equivalent stone diameter.

A second basic approach to predicting Kt’s was also outlined using two variables only related to the rubble-mound’s characteristics, i.e. the relative height, hc/ds, and  the relative width, At/hcd50.  The height and width (H –W) approach ignores the influence of wave height and period.  Although these two variables do not match with the dominant mode analysis they can make reasonable estimates of Kt’s for an entire data set and the role of the variables is unambiguous.  For the three data sets considered in detail the H – W approach explained about 80 to 90% of the variance in the Kt’s and provides an objective comparison of data trends.  It is hoped that the H –W approach can be used to compliment the dominant mode approach to provide better insight on the transmission process and better prediction of Kt’s.

References

Ahrens, J.P.,(1987a). “Characteristics of Reef Breakwaters”,  U.S. Army Corps of Engineers, Coastal Engineering Research Center, Technical Report CERC-87-17, Waterways Experiment Station, Vicksburg, MS.

Ahrens, J.P., (1987b). “Reef Breakwater Response to Wave Attack”, Proc. ASCE Conf. On Berm Breakwaters: Unconventional Rubble-Mound Breakwaters, Ottawa, Canada, Willis, Baird, & Magoon – Editors, 22 – 40.

Ahrens, J.P., and Fulford, E.T., (1988). “Wave Energy Dissipation by Reef Breakwaters”, Proc. Oceans ’88 Conf., Baltimore, MD, 1244 - 1249.

Ahrens, J.P., and Cox, J.,(1990). “Design and Performance of Reef Breakwaters”, Journal of Coastal Research, Special Issue 7, Fort Lauderdale, FL,  61 –75.

Allsop, N.W.H., (1983). “Low-crest breakwater, studies in random waves”, Conf. Coastal Structures ’83, Arlington, VA, 94 – 107.

Cornett, A., Mansard, E., and Funke, E. (1993). “Wave Transformation and Load Reduction Using a Small Tandem Reef Breakwater – Physical Model Tests”, Proc. Waves ’93 Conf., New Orleans, LA, 1008 – 1023.

Daemen, I.F.R., (1991). “Wave Transmission at Low-Crested Structures”, Master’s Thesis Delft U. of Technology, Delft Hydraulics H 462, Delft, the Netherlands.

Daniel, C., and Wood, F.S., (1980). Fitting Equations to Data: Computer Analysis of Multifactor Data, 2nd Edition, John Wiley & Sons, New York, 458 pages.

van Gent, M.R.A., (1993) “Stationary and Oscillatory Flow through Coarse Porous Media”, Communication on Hydraulic and Geotechnical Engineering, Report No. 93 – 9, Delft U. of Technology, Delft, the Netherlands. about 130 pages with appendices

van der Meer, J.W., (1988). “Rock Slopes and Gravel Beaches under Wave Attack”, PhD Thesis Delft U. of Technology, Delft Hydraulics No. 396, Delft, the Netherlands

van der Meer, J.W., and d’Angremond, K. (1991). “Wave Transmission at Low-Crested Structures”, Proc. ICE Conf. on Coastal Structures and Breakwaters, London, Thomas Telford, 25 – 40 and discussion 43 – 54.

van der Meer, J.W., and Pilarczyk, K.W., (1995). “Low-Crested Rubble Mound  Structures”, Proc. of Workshop on River, Coastal and Shoreline Protection, Fort Collins, CO., editors Thorne, Abt, Barends, Maynord & Pilarczyk,  John Wiley & Sons, Chichester, U.K., 233 – 249.

Powell, K.A., and Allsop, N.W.H., (1985). “Low-Crest Breakwaters, Hydraulic Performance and Stability”, Hydraulics Research Wallingford, Report S 57, Wallingford, United Kingdom.

Seelig, W.N., (1980). “Two-Dimensional Tests of Wave Transmission and Reflection Characteristics of Laboratory Breakwaters”, U.S. Army Corps of Engineers, Coastal Engineering Research Center, Technical Report 80 – 1, Fort Belvoir, VA.

Sheppard, D.M., and Hearn, J.K., (1989). “Performance and Stability of Low- Crested Breakwaters”, U. of Florida Report UFL/COEL – 88/004, Gainesville, FL.

Shore Protection Manual (1984).Vol.II, U.S. Army Corps of Engineers, Coastal Engineering Research Center, Vicksburg, MS.

Wiegel, R.L., (1964). Oceanographical Engineering , Prentice-Hall, Inc, Englewood Cliffs, NJ.

## Figures

### Fig. 1. Multilayer rubble-mound breakwater cross section, see the SPM Figures 7-116

and 7-117.  (this figure is not included)

### Fig. 2. Data from HRS Wallingford.

Fig. 3 Cross sectional profile of a reef breakwater, as built and after damage by wave action, See Figure 3, Ahrens (1987a). (this figure is not included)

Fig. 4. Conceptual model of the dominant mode of wave transmission over and through a rubble-mound breakwater.

Fig. 5. Predicted Kt (through) by Eq. 4 versus observed data from a number of sources.

Fig. 6. Predicted Kt(over) by Eq. 7 versus data of van der Meer (1988) and Daemen (1991).

Fig. 7. Predicted Kt (over and through) using Eqs. 7 and 4 versus data of van der Meer (1988) and Daemen (1991).

Fig. 8. Predicted Kt (over) using Eq. 7 versus data of Seelig (1980).

Fig. 9. Predicted Kt (over and through) using Eqs. 7 and 4 versus the data of Seelig (1980).

Fig. 10. Predicted Kt for submerged rubble-mounds using Eq.s. 4 and 8 versus data of Seelig (1980).

Fig. 11. Predicted Kt for submerged rubble-mounds using Eq.s. 4 and 8 versus data of  van der Meer (1988) and Daemen (1991).

Fig. 12. Ratio of predicted versus observed Kt as a function of relative freeboard, F/Hmo.

Predicted values computed using methods given in this paper.

Fig. 13. Summary of height and width characteristic of rubble-mound breakwaters from various studies.

Fig. 14.  Comparison of data sets for a variety of structure heights and widths as discussed in test.

Fig. 15. Comparison of data sets for a single structure with a varying water level (tide) as discussed in text.