May 5, 2010

# CMS-Wave Validation Effort at FRF

# Wave Data Inventory

Objective:  Inventory furnished wave gage data sets (forcing and observations) to assess proper files are being used to force select events and the associated observation data sets cover same time period, allowing proper validation and is suitable for IMEDS input.

## Background

Several data sets have been provided by Harry Friebel and Eve Devaliere.  Initial forcing files provided by Harry Friebel (E2_na.mat, E4_na.mat, E5_na.mat, E6_na.mat) on 3/24/2010.  Additional forcing and observation files were provided by Eve on 4/10/2010.  All data sets were binary Matlab arrays in the “wavedat” structure file format (see IMEDS Appendix).

Appendix 1: Wavedat Format Description

Data processed by IMEDS are stored in special structure called wavedat. The various preprocessors modify the given raw format (e.g.: WW3 or NDBC) into this wavedat format. A summary of the required wavedat input fields follows:

• file: (string) Processing History

• time: [1x m double] Observation Date and Time

• espt: {1x m cell} Directional Wave Spectra (m2/(hz x deg)

• dwfhz: [1x64 double] Frequency Bins (Hz)

• dwdeg: [1x24 double] Angle Bins (deg)

• dwAvv: {1x m cell} 1D Energy-Frequency Spectga

• hs: [1x m double] Significant Wave Height (m)

• fp: [1x m double] Peak Frequency (Hz)

• thetap: [1x m double] Peak Direction (Hz)

• winddir: [1x m double] Wind Direction (Deg From True North)

• windspeed: [1x m double] Wind Speed (m/s)

• name: (string) Data Set Name

• size: [1x1] Number of Records

• type: '2D' or 1D, Specifies Directional or Non-Directional Data

• lat: [1x1] Latitude (decimal degrees +N/-S)

• lon: [1x1] Longitude (decimal degrees +E/-W)

• depth: [1x1] Water Depth (m)

## FORCING DATA SETS

### Original ENG_*.mat (Harry Friebel, 3/24/2010)

4 Events (E2, E4, E5, E6)

- E2_na.mat (CDIP 430, 26m)

Max Hs =   1.7 m, Assoc Period  12.3 sec, Mean Ang = 110 degN

Start: 12-Jul-2008 00:25, Stop: 14-Jul-2008 23:55, TS =  0.50 hr, 139 Time Steps

- E4_na.mat (CDIP 430, 26m)

Max Hs =   2.2 m, Assoc Period  10.2 sec, Mean Ang =   325 degN

Start: 27-Sep-2008 12:55, Stop: 28-Sep-2008 12:55, TS =  0.50 hr, 49 Time Steps

- E5_na.mat (CDIP 430, 26m)

Max Hs =   3.4 m, Assoc Period  14.9 sec, Mean Ang =    70 degN (Angle Correct?)

Start: 20-Oct-2008 15:55, Stop: 21-Oct-2008 12:55, TS =  0.50 hr, 42 Time Steps

- E6_na.mat (CDIP 430, 26m)

Max Hs =   2.4 m, Assoc Period  14.9 sec, Mean Ang =  308 degN (Angle Correct?)

Start: 19-Feb-2009 07:54, Stop: 19-Feb-2009 14:54, TS = 0.50 hr, 15 Time Steps

### Second Set of Forcing (provided by Eve Devaliere, 4/10/2010)

6 Files with 26m (Events 2, 4, 5, and 6) and 17m Waverider (Event 4)

1) Event 2 (‘wvrdr430MonthlyWavedat.200807UTC.mat’)(CDIP 430,26m)

Max Hs =   1.7 m, Assoc Period  12.1 sec, Mean Ang = 110 degN

Start: 12-Jul-2008 00:24, Stop: 14-Jul-2008 23:54, TS = 0.50 hr, 144 Time Steps

2) Event 4 (‘wvrdr430.mat’)(CDIP 430, 26m)

Max Hs =   2.2 m, Assoc Period  10.4 sec, Mean Ang =   105 degN

Start: 27-Sep-2008 12:55, Stop: 28-Sep-2008 12:55, TS =  0.50 hr, 49 Time Steps

3) Event 5 (‘wvrdr430_oct08.mat’)(CDIP 430, 26m)

Max Hs =   3.3 m, Assoc Period  14.5 sec, Mean Ang =    68 degN

Start: 20-Oct-2008 15:55, Stop: 21-Oct-2008 12:55, TS =  0.50 hr, 42 Time Steps

- Event 6 (‘wvrdr430_Feb09.mat’)(CDIP 430, 26m)

Max Hs =   2.4 m, Assoc Period  14.5 sec, Mean Ang =   122 degN

Start: 19-Feb-2009 07:54, Stop: 19-Feb-2009 14:54, TS =  0.50 hr, 15 Time Steps

- Event 2 (‘wvrdr430MonthlyWavedat.200807UTC.mat’)(CDIP 430, 26m)

Max Hs =   1.7 m, Assoc Period  12.1 sec, Mean Ang = 110 degN

Start: 12-Jul-2008 00:24, Stop: 14-Jul-2008 23:54, TS = 0.50 hr, 144 Time Steps

- Event 4 (‘S1E3_na.mat’)(CDIP 430, 26m)

Max Hs =   5.2 m, Assoc Period  12.3 sec, Mean Ang =  83 degN

Start: 22-Sep-2008 00:25, Stop: 27-Sep-2008 23:55, TS = 0.50 hr, 283 Time Steps

- Event 4 (‘wvrdr3630.mat’)(CDIP 3630, 17m)

Max Hs =   1.9 m, Assoc Period  12.5 sec, Mean Ang =    87 degN

Start: 27-Sep-2008 13:00, Stop: 28-Sep-2008 13:00, TS =  1.00 hr, 25 Time Steps

### Observation Files (provided by Eve Devaliere, 4/10/2010)

Six (6) Events with Typically 6 Gages per event separated by folders per event

Events -  2, 3, 4, 5, 6, 7

Gages - 26mWvrdr, 17mWvrdr, 11mAwac, 8mAwac, 6mAwac, 5mAwac

Inner Grid

Outer Grid

Cross-Shore Array (AWACs)

17-m

Waverider

26-m

Waverider

Model Domains

- Event 2 (Long Northeaster July 2008)

Max Hs =   1.7 m, Assoc Period  12.5 sec, Mean Ang = 127 degN

Start: 01-Jul-2008 00:24, Stop: 31-Jul-2008 23:36, TS = 0.50 hr, 1485 Time Steps

- Event 3 (September 2008)

Max Hs =   5.2 m, Assoc Period  12.5 sec, Mean Ang =  83 degN

Start: 22-Sep-2008 00:55, Stop: 27-Sep-2008 23:55, TS = 1.00 hr, 142 Time Steps

- Event 4 (Hurricane Bill)

Max Hs =   2.2 m, Assoc Period  10.4 sec, Mean Ang =  105 degN

Start: 27-Sep-2008 12:55, Stop: 28-Sep-2008 12:55, TS = 0.50 hr, 49 Time Steps

- Event 5 (October 2008)

Max Hs =   3.3 m, Assoc Period  14.5 sec, Mean Ang =   68 degN

Start: 20-Oct-2008 15:55, Stop: 21-Oct-2008 12:55, TS = 0.50 hr, 42 Time Steps

- Event 6 (Short Nor’Easter February 2009)

Max Hs =   2.1 m, Assoc Period  10.4 sec, Mean Ang =   121 degN

Start: 19-Feb-2009 07:54, Stop: 19-Feb-2009 13:54, TS =  1.00 hr, 7 Time Steps

- Event 7 (August 2009)

Max Hs =   3.9 m, Assoc Period  18.1 sec, Mean Ang = 108 degN

Start: 22-Aug-2009 08:51, Stop: 23-Aug-2009 14:51, TS = 1.00 hr, 31 Time Steps

FROM CDIP Website

E3 - Max Hs = 5.2 m, Assoc Tp = 12.5 sec, Mean Ang = 85 degN

E5 Max Hs = 3.3 m, Assoc Tp = 14.5 sec, Mean Ang = 68degN

E6 - Max Hs = 2.1 m, Assoc Tp = 10.4 sec, Mean Ang = 121degN

E7 – Max Hs = 3.9 m, Assoc Tp = 18.1 sec, Mean Ang = 108degN