Lab data file: CHL_ideal_inlet_labdata.dat (more complete and corrected table)*

x – distance (cm)   d – water depth (cm)  H – wave height (cm)  U – current (cm/sec)

4 pairs (H,U) data for each x, 1st H is center, 2nd and 3rd are two sides, 4th is average

* x is cross-shore distance from offshore boundary (-9 m) to bayward boundary (9.6 m),

same as in the numerical wave grid; d is relative to mean water level; H is significant

wave height; U is the cross-shore current velocity; 0 value can mean either real 0 or no

data available.

Simplified data file (no txt): CHL_Run1-12.dat - for input to Matlab codes.  Added the

wave maker location and incident wave (input) height (marked as -900 m for x) for each experiment (in addition to CHL_ideal_inlet_labdata.dat).  Total 13 data points in each experiment, the 2nd to 12th locations corresponds to model saved stations 7 to 17.

Wave model version (CMS: Coastal Modeling System): CMS-Wave_v1.9official.exe

Basic files required:

*.sim (model input file list)

*.dep (bathymetry, land is 0 or negative)

*.eng (incident wave spectrum)

*.std  (model control parameters)

*.cur (background current field)**

** Background current field (optional file) is generated by CMS-Flow adjusting the water levels in the bay and office boundaries to match the measured current data.

When running CMS-Wave, to specify different breaking formula, a 15th parameter is needed in the 1st line *.std

0  (or not provided) -  extended Goda

1                             -  extended Miche

2                             -  Battjes and Janssen

3                             -  Chawla and Kirby

Input files for CHL-98-31 run cases:

Runs 1-4 (without current):

chl9831lab1-4.sim

chl9831lab1-4.dep

chl9831lab1-4.std (breaking formula assigned by the 15th parameter)

chl9831lab1-4.eng (incident wave based on average seaward gauges 1-7)

Runs 5-8(with weak current):

chl9831lab5-8.sim

chl9831lab5-8.dep

chl9831lab5-8.std

chl9831lab5-8.eng

chl9831lab5-8.cur (same current fields for Runs 5-8)

Runs 9-12(with strong current):

chl9831lab9-12.sim

chl9831lab9-12.dep

chl9831lab9-12.std

chl9831lab9-12.eng

chl9831lab9-12.cur (same current fields for Runs 9-12)

All runs are done in the laboratory scale.

We have added classical Miche’s breaking (H/d<0.64 tanh kd / kd) as a cap over all formulas – otherwise, not enough dissipation for some formulas in certain conditions.

Results saved as *.wav, *.obs, and *.out files.  Three matlab programs are attached for showing model results at saved locations (in *.std): plot1to4.m, plot5to8.m and plot9to12m – see plot1to4.tif etc.